CREA · Centre for Research on Energy and Clean Air

Identifying Systematically Irregular

PM2.5 Reporting Stations

A Multi-Method Consensus Approach Across Four Countries

Data Challenge Submission

China · Germany · India · United States  |  OpenAQ 2021–2023

# Executive Summary

This study identifies PM2.5 monitoring stations whose reporting behaviour is systematically irregular — potentially under-reporting, over-reporting, or failing to report — across four national networks drawn from OpenAQ (approximately 42 million hourly observations, 2021–2023, 3,151 stations).

Rather than relying on a single detector, we apply three independent methods — spatial Z-score (station vs. neighbours), Isolation Forest (multivariate behaviour), and Benford's Law (digit-pattern integrity) — and combine them into a consensus ranking. The premise, following majority-voting frameworks in the sensor-network literature, is that stations flagged by multiple independent methods constitute far stronger evidence than any single method alone.

Key findings: Two stations were flagged by all three methods; 28 by two methods. The strongest candidate (station 3033a, China) shows consistent evidence of under-reporting: it reports far below its spatial neighbours (Z = −8.8) with an unnatural digit distribution (Benford deviation 5.5 SD). Across the Z-score results, under-reporting dominates in China (55 stations), consistent with documented concerns in the air-quality literature.

Critical caveat: Benford conformity varies sharply by country — acceptable for India, marginal for China, but non-conforming for Germany and the USA. Benford-based signals are therefore reliable only for India and China; for Germany and the USA, spatial and multivariate evidence must carry the conclusion.

# 1. Introduction

Fine particulate matter (PM2.5) is among the most health-damaging air pollutants, directly informing public-health policy and regulatory action. The integrity of the monitoring data underlying that policy is therefore essential. Large-scale networks, however, are vulnerable to both technical faults and — the focus of this work — systematically irregular reporting.

Existing anomaly-detection frameworks for air-quality networks, notably the ADF framework of Chen et al. (2018), focus predominantly on technical sensor faults — indoor placement, emission-source proximity, hardware malfunction. This study extends that lineage toward a distinct question: identifying stations whose reporting is irregular in ways consistent with systematic misreporting, a dimension the original frameworks do not directly address.

# 2. Data

The analysis draws on OpenAQ measurement data for four countries spanning 2021–2023. After cleaning and filtering, the working dataset comprises 3,151 stations across China, Germany, India, and the USA.

Preparation (NB1–NB2). Sentinel values and implausible coordinates were removed; a completeness filter (≥50% of the active reporting window) was applied, justified by all four countries showing first-quartile completeness well above that threshold. A left-truncation correction was introduced so stations that began reporting mid-window (notably in China) are assessed against their active period rather than the full window, avoiding a systematic bias against newer stations.

A key structural finding. A Kolmogorov–Smirnov test confirmed that the four countries' PM2.5 distributions differ significantly (p < 0.05). This result motivates a per-country design throughout: every detection method is applied within each country, never on pooled data.

# 3. Methodology

Design principle — no orphan parameters. Every threshold used is either derived empirically from the data (with documented provenance) or drawn from a citable standard. No parameter is set by convention alone.

## 3.1 Spatial Z-score with leave-one-out

Each station's mean PM2.5 is compared to the mean of its spatial neighbours (within a per-country radius). Crucially, the neighbour statistics are computed excluding the target station itself (leave-one-out), preventing an anomalous station from contaminating its own benchmark. The suspicion threshold was validated against the data rather than assumed. Anomalies are decomposed into SL (significantly low → under-reporting) and SH (significantly high → over-reporting), following Chen et al. (2018).

## 3.2 Isolation Forest

A multivariate detector run per country over seven engineered station features (mean, variability, range, missingness, extreme-rate relative to the country-specific P99, near-zero rate, and lag-1 autocorrelation). The contamination parameter (0.05) was validated empirically through a sensitivity analysis of P99-exceedance proportions, not accepted as a default.

## 3.3 Benford's Law

First-digit frequencies are compared to the Benford distribution. Following the principle that conformity must be proven before irregularities can be attributed, global and per-country conformity are validated first (Section 4.2); only then are per-station deviations scored, and always as a relative signal within each country.

## 3.4 Consensus

Each method's scores are rank-normalised within country (placing all methods on a common 0–1 scale, robust to outliers), then combined two ways: a vote count (how many methods flag a station) and a combined score (mean normalised score). The two are complementary — the vote count answers “how many agree,” the combined score “how strong overall.”

# 4. Results

## 4.1 Per-method summary

The spatial Z-score flagged 129 stations overall (76 under-reporting, 53 over-reporting), with under-reporting concentrated in China (55 stations). Isolation Forest and Benford each contributed independent signals, detailed below.

## 4.2 Benford conformity by country — a finding in itself

Before per-station scoring, global conformity was confirmed (MAD = 0.006, close conformity). Per-country conformity, however, diverges sharply:

| Country | MAD | Verdict | Benford reliability |
| --- | --- | --- | --- |
| India | 0.009 | Acceptable | Reliable |
| China | 0.013 | Marginal | Fairly reliable |
| Germany | 0.017 | Non-conforming | Use with caution |
| USA | 0.025 | Non-conforming | Interpret very cautiously |

Implication: For Germany and the USA, even honest data does not follow Benford's Law; a Benford flag there cannot by itself indicate manipulation. For these countries, the spatial and multivariate methods carry the conclusion.

## 4.3 Inter-method correlation

Correlations among the three normalised scores are very low (0.05–0.11), confirming the methods capture genuinely different signals. This strengthens the value of consensus — agreement is not redundancy — but also makes unanimous (3-of-3) agreement rare and correspondingly strong.

## 4.4 Consensus ranking

| Rank | Station | Country | Direction | Votes | Score |
| --- | --- | --- | --- | --- | --- |
| 1 | 3033a | China | SL under-report | 3 / 3 | 0.98 |
| 2 | 3248a | China | — | 3 / 3 | 0.97 |
| 3 | 2598a | China | Benford (small n) | 2 / 2 | 0.998 |
| 6 | airnow_261130001 | USA | — | 2 / 2 | 0.98 |
| 7 | site_5856 | India | — | 2 / 3 | 0.97 |

Manual review — station 3033a (rank 1). Flagged by all three methods with mutually consistent evidence: reports well below neighbours (under-reporting) with an unnatural digit distribution. This is the strongest manipulation candidate and the priority for follow-up verification.

Manual review — station 2598a (rank 3). Highest Benford deviation overall (8.9 SD), but based on only ~3,250 observations versus ~8,000 for comparable Chinese stations. The extreme score may be partly a small-sample artefact; its time series must be inspected before any conclusion.

# 5. Discussion & Limitations

The consensus approach converts three imperfect, independent signals into a defensible ranking. Its strength is also the source of its principal limitation: a consensus is only as good as the methods composing it.

- Shared temporal blind spot. The Z-score and Isolation Forest baselines use per-station aggregates, discarding the time dimension. Stations that under-report only intermittently may evade both; a spatio-temporal extension is the natural next step.

- Country-dependent Benford reliability. As shown in 4.2, Benford is not dependable for Germany and the USA. Conclusions there rest on the other two methods.

- Small-sample sensitivity. High deviation scores at low observation counts (e.g., station 2598a) may be partly artefactual and require manual inspection.

- Consensus filters, it does not prove. The ranking prioritises candidates for investigation; it is not itself proof of manipulation. Field or administrative verification remains necessary.

- Fire cross-reference (GFED4). Over-reporting (SH) candidates may be genuine — reflecting real smoke events. A GFED4 cross-reference notebook applies directional logic (exonerate SH-with-fire, retain SL regardless) and should be run with the actual fire database before final conclusions.

# 6. Conclusions & Recommendations

Three independent detection methods, combined by consensus, identify a small set of PM2.5 stations warranting closer scrutiny. The strongest evidence points to under-reporting among a subset of Chinese stations, led by station 3033a.

Recommendation. Prioritise verification of the multi-method (vote ≥ 2) candidates in India and China, where all three methods are reliable, beginning with unambiguous under-reporting cases that fire activity cannot explain. Treat Germany/USA Benford flags as provisional pending spatial-temporal confirmation.

# Appendix · Reproducibility & References

Pipeline. NB1 (cleaning) → NB2 (EDA & parameter derivation) → NB3 (spatial Z-score) → NB4 (Isolation Forest) → NB5 (Benford) → NB6 (consensus) → NB7 (GFED4 cross-reference). All parameters are centralised in params.yml with provenance recorded in parameter_derivation.json.

Core references.

- Chen, L.-J., et al. (2018). ADF: An Anomaly Detection Framework for Large-scale PM2.5 Sensing Systems. IEEE IoT Journal 5(2).

- Liu, F.T., Ting, K.M., Zhou, Z.-H. (2008). Isolation Forest. IEEE ICDM.

- Nigrini, M.J. (2012). Benford's Law: Applications for Forensic Accounting, Auditing, and Fraud Detection. Wiley.

- Fu et al. (2014). Application of Benford's Law to air-quality (AQI) data.

- AIrSense (2023). Anomaly Detection and Repairing for Improving Air Quality Monitoring. Sensors 23(2).